Packet Tracer - Explore a NetFlow Implementation

Objectives

Part 1: Observe NetFlow Flow Records - One Direction

Part 2: Observe NetFlow Records for a Session that Enters and Leaves the Collector

Background / Scenario

In this activity, you will use Packet Tracer to create network traffic and observe the corresponding NetFlow flow records in a NetFlow collector. Packet Tracer offers a basic simulation of NetFlow functionality. It is not a replacement for learning NetFlow on physical equipment. Some differences may exist between NetFlow flow records generated by Packet Tracer and by records created by full-featured network equipment.

Instructions

Part 1: Observe NetFlow Flow Records - One Direction

Step 1: Open the NetFlow collector.

- From the NetFlow Collector, click the Desktop tab. Click the Netflow Collector icon.

- Click the On radio button to activate the collector as necessary. Position and size the window so that it is visible from the Packet Tracer topology window.

  - Click PC-1.

  - Open the Desktop tab and click the Command Prompt icon.

  - Enter the ping command to test connectivity to the default gateway at 10.0.0.1.

    - After a brief delay, the NetFlow Collector screen will display a pie chart.

    - Click either the pie chart or the legend entry to display the flow record details.

    - The flow record will have entries similar to those in the table below. Your timestamps will be different.

    - Click PC-2 > Desktop.

    - Open a command prompt and ping the default gateway 10.0.0.1.

    - Return to PC-1 and repeat the ping to the gateway.

    - Issue pings from PC-3 and PC-4 to the default gateway address.

    - Click NetFlow Collector > Physical tab.

    - Click the red power button to turn off the server. Then click it again to turn the server back on. (Note: You may need to scroll over or zoom out.)

    - From the NetFlow Collector, click the Desktop tab.

    - Click the Netflow Collector icon. Click the “On” radio button to activate the collector. Close the NetFlow Collector window.

    - Before you access a web server from PC-1, predict how many flows will there be in the pie chart? Explain.

    - Click PC-1 > Desktop. Close the Command Prompt window, if necessary. Click the Web Browser icon.

    - the Web Browser for PC-1, enter 192.0.2.100 and click Go. The Example Website webpage will display.

    - After a short delay, a new pie chart will appear in the NetFlow collector. You will see at least two pie segments for the HTTP request and response. You might see a third segment if the ARP cache for PC-1 timed out.

    - Click each HTTP pie segment to display the record and verify your predictions.

    - Click the link to the Copyrights page.

    - Power cycle the NetFlow Collector to clear the flows.

    - Turn on the Netflow Collector service.

    - Before you access the Web Server by its URL.

    - 1, enter www.example.com in the URL field and press Go.

    - After the flows are displayed, inspect each flow record.

Step 2: Ping the default gateway from PC-1.

C:\> ping 10.0.0.1

Note: The first set of pings may not be sent to the NetFlow Collector because the ARP process must first resolve IP and MAC addresses. If after 30 seconds, a pie chart does not appear, ping the default gateway again.

| Entry | Value | Explanation |
| --- | --- | --- |
| Traffic contribution | 100% (1/1) | This is the proportion of all traffic represented by this flow. |
| IPV4 SOURCE ADDRESS | 10.0.0.10 | This is the source IP address of the flow packets. |
| IPV4 DESTINATION ADDRESS | 10.0.0.1 | This is the destination IP address of the flow packets. |
| TRNS SOURCE PORT | 0 | This is the transport layer source port. The value is 0 because this is an ICMP flow. |

| Entry | Value | Explanation |
| --- | --- | --- |
| TRNS DESTINATION PORT | 0 | This is the transport layer destination port. The value is 0 because this is an ICMP flow. |
| IP PROTOCOL | 1 | This identifies Layer 4 service, typically 1 for ICMP, 6 for TCP, and 17 for UDP. |
| timestamp first | 00:47:49.593 | This is the timestamp for the beginning of the flow. |
| timestamp last | 00:47:52.598 | This is the timestamp for the last packet in the flow. |
| tcp flags | 0x00 | This is the TCP flag value. In this case, no TCP session was involved because the protocol is ICMP. |
| counter bytes | 512 | This is the number of bytes in the flow. |
| counter packets | 4 | This is the number of packets in the flow. |
| interface input | Gig0/0 | This is the interface of the flow exporter that collected the flow in the input direction (into the monitoring device interface). |
| interface output | Null | This is the interface of the flow exporter that collected the flow in the output direction (out of the monitoring device interface). The value is “Null” because this was a ping to the input interface. |

In this case, the flow represents the ICMP ping from host 10.0.0.10 to 10.0.0.1. Four ping packets were in the flow. The packets entered interface G0/0 of the exporter.

Note: In this activity, the Edge router has been configured as a NetFlow flow exporter. The LAN interface is configured to monitor flows that enter it from the LAN. The serial interface has been configured to collect flows that enter it from the internet. This has been done to simplify this activity.

To see traffic that matches a full bi-directional session, the NetFlow exporter would need to be configured to collect flows entering and leaving a network.

Step 3: Create additional traffic.

Question:

What do you expect to see in the NetFlow collector flow records? Will the statistics for the existing flow record change, or will a new flow appear in the pie chart?

Type your answers here.

A new flow record will appear in the pie chart.

NetFlow defines a flow as a unidirectional set of packets that share the same:

- Source IP address

- Destination IP address

- Source port

- Destination port

- IP protocol

Because PC-2 has a different source IP address than PC-1, its ICMP ping traffic does not match the existing flow record. As a result, the NetFlow collector creates a separate flow entry, shown as a new color-coded segment in the pie chart.

Question:

How will this traffic be represented? As a new segment in the pie chart or will it modify the values in the existing flow record?

Type your answers here.

The traffic from PC-1 will modify the values in the existing flow record, not create a new segment.

Since the second ping from PC-1 uses the same source IP, destination IP, protocol (ICMP), and ports, NetFlow treats it as part of the same flow.

As a result:

- The packet counter increases

- The byte counter increases

- The traffic contribution percentage grows

- The flow segment in the pie chart becomes larger

However, the flow does not become a new entry because the flow identity remains unchanged.

What should happen to the display in the flow collector?

Type your answers here.

A new flow record should appear for each PC.

Each PC has a unique IP address, so each ICMP ping generates a distinct unidirectional flow. The NetFlow collector will therefore:

- Add one new pie-chart segment per PC

- Display separate flow records for PC-3 and PC-4

- Show multiple colored portions in the pie chart representing traffic from different hosts

This visually demonstrates how NetFlow tracks traffic on a per-source basis.

Part 2: Observe NetFlow Records for a Session that Enters and Leaves the Collector

The NetFlow exporter has been configured to collect flows that exit the LAN and enter the router from the Internet.

Step 1: Access the Web Server by IP Address.

Before continuing, power cycle the NetFlow Collector to clear the flows.

Type your answers here.

From your knowledge of network protocols and NetFlow, predict the values for the web page requests leaving the LAN.

| Record Field | Value | Guidelines |
| --- | --- | --- |
| Source IP address | 10.0.0.10 | N/A |
| Destination IP address | 192.0.2.100 | N/A |
| Source Port | 1025–5000 (MS Windows default, which is what PT uses. | This is an approximate value that is dynamically created. |
| Destination Port | 80 | N/A |
| Input Interface | Gig0/0 | N/A |
| Output Interface | Se0/0/1 | N/A |

Predict the values for the web page reply entering the NetFlow exporter router from the internet.

| Record Field | Value | Guidelines |
| --- | --- | --- |
| Source IP address | 192.0.2.100 | N/A |
| Destination IP address | 10.0.0.10 | N/A |
| Source Port | 80 | N/A |
| Destination Port | 1025-5000 | This is whatever value was randomly assigned form the ephemeral port range. |
| Input Interface | Se0/0/1 | N/A |
| Output Interface | Gig0/0 | N/A |

Questions:

What happened? Explain. (Hint: compare the port number on the host for the flows.)

Type your answers here.

Two new NetFlow records were created.

When the user clicked the Copyrights link, the browser opened a new HTTP connection to the same web server. Even though the destination IP and port remained the same, the client selected a new ephemeral source port for the new request.

Because NetFlow treats each unique 5-tuple (source IP, destination IP, source port, destination port, protocol) as a new flow, this resulted in:

- One new flow for the HTTP request

- One new flow for the HTTP response

This demonstrates how modern browsers create multiple TCP sessions even when connecting to the same server.

Compare the flows. Aside from the obvious timestamp, source and destination IP address, port, and interfaces, differences, what else is different between the request and response flows?

Type your answers here.

The TCP flags are different between the request and response flows.

- The request flow shows a TCP flag value of 0x02, which represents a SYN packet.
→ This indicates that the client is initiating a TCP connection.

- The response flow shows a TCP flag value of 0x12, which represents a SYN-ACK packet.
→ This indicates that the server has acknowledged the connection request and is completing the TCP three-way handshake.

These flag differences reflect the direction and role of each packet in the TCP session setup process.

Step 2: Access the Web Server by URL.

Question:

What do you think you will see in the NetFlow collector display?

Type your answers here.

You will see four flows in total.

This is because accessing a website by URL requires an additional DNS lookup before the HTTP session can begin. The four flows are:

- DNS request (UDP) from PC-1 to the DNS server

- DNS response (UDP) from the DNS server to PC-1

- HTTP request (TCP) from PC-1 to the web server

- HTTP response (TCP) from the web server to PC-1

NetFlow records each of these as a separate unidirectional flow, resulting in four pie-chart segments.

Question:

What values do you see for the IP protocol field of the flow record? What do these values mean?

Type your answers here.

The IP protocol field shows the values 6 and 17.

- 6 = TCP (Transmission Control Protocol)
→ Used for HTTP traffic on port 80 because reliable, connection-oriented delivery is required for web content.

- 17 = UDP (User Datagram Protocol)
→ Used for DNS queries and responses because DNS prioritizes speed and low overhead over guaranteed delivery.

These values match the standard IP protocol number assignments and confirm the types of traffic being observed.

End of document