Packet Tracer - Logging from Multiple Sources

Objectives

Part 1: Use syslog to capture log files from multiple network devices

Part 2: Observe AAA user access logging

Part 3: Observe NetFlow information

Background / Scenario

In this activity, you will use Packet Tracer to view network data generated by syslog, AAA, and NetFlow.

Instructions

Part 1: View Log Entries with Syslog

Step 1: The syslog Server

Syslog is a messaging system designed to support remote logging. Syslog clients send log entries to a syslog server. The syslog server concentrates and stores log entries. Packet Tracer supports basic syslog operations and can be used for demonstration. The network includes a syslog server and syslog clients. R1, R2, Core Switch, and the Firewall are syslog clients. These devices are configured to send their log entries to the syslog server. The syslog server collects the log entries and allows them to be read.

Log entries are categorized by seven severity levels. Lower levels represent more serious events. The levels are: emergencies (0), alerts (1), critical (2), errors (3), warnings (4), notifications (5), informational (6), and debugging (7). Syslog clients can be configured to ship log entries to syslog servers based on the severity level.

- Click the Syslog Server to open its window.

- Select the Services tab and select SYSLOG from the list of services shown on the left.

- Click On to turn on the Syslog service.

  - Click R1 > CLI tab.

  - Press Enter to get a command prompt and enter the command enable.

  - Enter the command debug eigrp packets to enable EIGRP debugging. The command line console will immediately fill with debug messages.

  - Return to the Syslog Server window. Verify that log entries appear on the syslog server.

  - After a few messages have been logged, click the radio button to turn the syslog service Off.

  - Close the R1 device window.

  - Click the Syslog Server to open its window.

  - Select the Desktop tab and select AAA Accounting. Leave this window open.

  - Click R2 > CLI.

  - Press Enter to get a command prompt. R2 will ask for username and password before granting access to its CLI. Enter the following user credentials: analyst and cyberops as the username and password, respectively.

  - Return to the Syslog Server’s AAA Accounting Records window.

  - R2, enter the logout command.

  - Click the Syslog Server to bring up its window. Close the AAA Accounting Records window.

  - From the Desktop tab, select Netflow Collector. The NetFlow collector services should be turned on.

  - From any PC, ping the Corp Web Server at 209.165.200.194. After a brief delay, the pie chart will update to show the new traffic flow.

- Syslog entries coming from syslog clients will be shown in the window on the right. Currently, there are no entries.

- Keep this window open and visible and move on to Step 2.

Step 2: Enable Syslog.

The devices are already configured to send log messages to the syslog server, but Packet Tracer only supports the logging for the debugging severity level with syslog. Because of that, we must generate debug level messages (level 7) so they can be sent to the syslog server.

Question:

What is some of the information that is included in the syslog messages that are being displayed by the Syslog Server?

Type your answers here.

The syslog messages contain detailed operational information generated by the network devices. From the EIGRP debug messages, you can see:

- Protocol type: For example, EIGRP, which identifies the routing protocol that generated the log.

- Packet type: Such as HELLO packets, which are used by EIGRP routers to discover and maintain neighbors.

- Interface name: The interface on which the packet was sent or received (e.g., GigabitEthernet0/0).

- Autonomous System (AS) number: Identifies the EIGRP routing domain (e.g., AS 1).

- Flags and sequence numbers: Internal protocol values used to manage communication reliability.

- Timestamp: Shows exactly when the event occurred.

- Source device: Indicates which router generated the message.

This information is useful for troubleshooting routing issues, monitoring protocol behavior, and verifying that routing updates are being exchanged correctly.

Part 2: Log User Access

Another important type of log relates to user access. Having records of user logins is crucial for troubleshooting and traffic analysis. Cisco IOS supports Authentication, Authorization and Accounting (AAA). With AAA, it is possible not only to delegate the user validation task to an external server but also to log activities.

TACACS+ is a protocol designed to allow remote authentication through a centralized server.

Packet Tracer offers basic AAA and TACACS+ support. R2 is also configured as a TACACS+ server. R2 will ask the server if that user is valid by verifying username and password, and grant or deny access based on the response. The server stores user credentials and is also able to log user login transactions. Follow the steps below to log in to R2 and display the log entries related to that login:

Question:

What information is in the log entry?

Type your answers here.

The AAA accounting log entry records important security and auditing details about the user login event. It includes:

- Date and time: The exact timestamp when the login occurred.

- Username: The account name used to access the router (analyst).

- Caller ID: The access method or terminal used (e.g., console connection).

- Flag value: Indicates the type of event — in this case, Start, meaning a user session has begun.

- NAS IP address: The IP address of the router that was accessed (R2).

- NAS port: The specific interface or port used for the login (con0).

This information allows administrators to track who logged in, when they logged in, and from which device, which is essential for security monitoring and compliance auditing.

Question:

What happened in the AAA Accounting window?

Type your answers here.

A new accounting record appeared with a Stop flag.

This new entry indicates that the user session has ended. When combined with the earlier Start entry, it allows an administrator to determine:

- The duration of the user session

- Whether the user properly logged out

- The full timeline of the login activity

These paired Start and Stop records are important for auditing, troubleshooting user access problems, and detecting suspicious behavior, such as sessions that never properly terminate.

Part 3: NetFlow and Visualization

In the topology, the Syslog server is also a NetFlow collector. The firewall is configured as a NetFlow exporter.

Note: The pie charts displayed will vary based on the traffic on the network. Other packets flows, such as EIGRP-related traffic, are being sent between devices. NetFlow is capturing these packets and exporting statistics to the NetFlow Collector. The longer NetFlow is allowed to run on a network, the more traffic statistics will be captured.

Reflection

While the tools presented in this activity are useful, each one has its own service and may need to run on totally different devices. A better way, explored later in the course, is to have all the logging information concentrated under one tool, allowing for easy cross-reference and powerful search capabilities. Security information and event management (SIEM) platforms can gather log files and other information from diverse sources and integrate the information for access by a single tool.

End of document